Aerodynamics of an airfoil at ultra-low Reynolds number

-----------------------------------------

1Md. Mahbub Alam （）

Harbin Institute of Technology, Shenzhen Graduate School, Shenzhen, China

Email: alamm28@yahoo.com

2Y. Zhou

Department of Mechanical Eng., The Hong Kong Polytechnic University, Hong Kong

Md. Mahbub Alam1, Y. Zhou2

Abstract   A comprehensive experimental study is conducted on the aerodynamic characteristics of a NACA0012 airfoil over a large range of angle () of attack and low- to ultra-low cord Reynolds numbers, 5.3×103 - 5.1×104, which is of both fundamental and practical importance. While the mean and fluctuating lift and drag coefficients were measured using a load cell, the detailed flow structure is documented using particle image velocimetry and laser-induced fluorescence flow visualization. CD increases monotonically from  = 0° to 90°, whilst CL grows from 0 to its maximum at  = 45° and then drops. The near wake characteristics examined were found to be consistent with the force measurements, including the vortex formation length, wake width, spanwise vorticity, wake bubble size and wavelength of K-H vortices.

1.1 Introduction

The aerodynamic characteristics of airfoils at a chord Reynolds number (Rec= cU/, where  and  are the density and viscosity of the fluid, respectively, and c is the chord length of an airfoil) of less than 5×105 are becoming increasingly important from both fundamental and industrial point of view [1]. To our knowledge, the lowest Rec, at which airfoil lift and drag coefficient (CL and CD) data are available, is 1.0×104. General researches on airfoil aerodynamics have focused on conventional aircraft design with Rec beyond 5×105 and the angle () of attack below stall. Mueller and DeLaurier (2003) reviewed the available low Rec studies, with almost all the measured Rec higher than the wind turbine values quoted above. Aerodynamics of airfoils operated on the condition of a low Rec (<105) has recently gained an increasing practical importance, with a variety of applications such as micro air vehicles (MAVs), unmanned air vehicles (UAVs), small wind turbines, and low-speed/high-altitude aircraft (Lin and Pauley 1996). The work aims to determine the dependence on  of CL and CD of NACA 0012 airfoil at Rec = 5.3×103 ~5.1×104.

1.2 Experimental Details

Experiments were conducted in a closed-loop water tunnel, with a test section of 0.3m (width) × 0.6m (height) × 2.4m (length). The flow speed in the test section ranges from 0.05 m/s to 4 m/s. NACA0012 airfoil was used as the test model with a chord length of c =  0.1 m and a span of 0.27 m. The tests were carried out at Rec = 5.3×103 ~ 5.1×104, over which the free-stream turbulence was between 0.4% and 0.5%.  varied from 0º to 90º with an increment step of 10º. CL and CD were measured using a 3-component load cell (Kistler 9251A) and the detailed flow structure is documented using particle image velocimetry (PIV) and laser-induced fluorescence flow visualization.

Fig. 1.1 Dependence of CL on 

1.3 Results and Discussion

1.3.1 Mean Drag and Lift

Figure 1 presents the load-cell measured CL and CD. Note that the data at Rec = 5.1×104 is only shown for α ≤ 40º because the force at α > 40º is relatively large, exceeding the valid range of the load cell used. Evidently, CL is dependent on Rec for all α, growing with higher Rec. The figure shows the occurrence of the stall at about 10° for Rec = 1.05×104 and 5.1×104. With an increment of α = 10°, the stall should occur beyond 10° slightly. Interestingly, the stall is absent at Rec = 5.3×103, CL climbing until   45°, without any rapid drop as at other Rec. Beyond the stall α, CL displays a maximum at α  45°, regardless of Rec. CD increases monotonously with  and reaches maximum at α =90°. CD is between 1.35 and 1.48, depending on Rec, at α = 90° where the airfoil is like a flat plate normal to incident flow.

1.3.2 Flow Structure

Figure 2 presents typical photographs of the flow structure at α = 10˚ ~ 90˚ for Rec = 5.3×103. At very small α, flow was fully attached (not shown). With increasing α (< 10˚), the suction side boundary layer separated initially near the trailing edge, and the separation point moved gradually toward the leading edge. At the same time, CL grew significantly, so did CD, though at a lower rate. At α = 10˚, the separated boundary layer was laminar for a rather long longitudinal distance, without reattachment (Fig. 2a), and with further increase in α remained separated (Fig. 2b).

| (a) | (b) | (c) | (d) | (e) | (f) |
| --- | --- | --- | --- | --- | --- |

Fig. 1. 2 Typical photographs from LIF flow visualization at (a) α = 10º, (b) 20º, (c) 40º, (d) 50º, (e) 70º, (f) 90º. Rec = 5.3×103

1.3.3 Velocity Field

Figure 3 presents the contours of the mean streamwise velocity, U, for different  (Rec = 5.3×103), where the superscript ‘*’ stands for normalization by c and/or U. The minimum U and the recirculation bubble size, enclosed by U = 0, provide a measure for the strength of recirculation. Figure 4 summarizes the dependence of Umax, Umin and the maximum bubble width  on α. Umax in the leading-edge shear layer is higher than in the trailing edge. The recirculation size grows with increasing α, along with Umax and Umin; their growth rate is higher at  < 45˚ than at  > 45˚. Umax at the trailing edge is lower than at the leading edge, consistent with the positive CL. Fitting each set of data in Fig. 4 to a second-order polynomial curve and to a sine curve, respectively, using the least square technique yields empirical correlations given in the figure.

1.4 Conclusions

The stall of an airfoil characterized by a rapid drop in CL occurs at about  10° at Rec = 1.05×104 and 5.1×104; the stall is however absent at Rec = 5.3×103, because the separated boundary layer remains laminar and separated for a sufficiently long downstream distance, without reattachment. CD increases monotonically from  = 0° to 90°, whilst CL grows from 0 to its maximum at  = 45° and then drops. With increasing , (i) the mean vortex formation length Lf (not shown) and Umin in the wake bubble diminish rapidly up to  = 45°; (ii) the wake width d, Umax, maximum mean vorticity (not shown), wake bubble width  and flow velocity Us (not shown) at separation all increase.

| (a) | (b) | (c) | (d) | (e) | (f) |
| --- | --- | --- | --- | --- | --- |

Fig. 1.3 Contours of the mean streamwise velocity, U = U/U∞: (a) α = 10º, (b) 20º, (c) 40º, (d) 50º, (e) 70º, (f) 90º. Rec = 5.3×103

(a)					(b)

Fig. 1.4 Dependence on    of the maximum streamwise velocity Umax in the shear layers separated from the leading and trailing edge, the minimum streamwise velocity Umin in the bubble and the bubble width . Solid and dashed lines represent the polynomial and sinusoidal fit to the measured data, respectively. In the equations, the unit of  is radian

1.5 References

Lin JCM, Pauley LL (1996) Low-Reynolds-number separation on an airfoil. AIAA J 34:1570-1577.

Mueller TJ,  DeLaurier JD (2003) Aerodynamics of small vehicles. Annu Rev Fluid Mech 35: 89-111.